FENCES

Below you will find a summary of code requirements for fence construction within the Village of Antioch, based upon the 2012 International Residential Code.

## WHERE CAN MY FENCE BE LOCATED?

- Fences not over six feet (6') in height may be constructed on any lot behind the front of any principal building located on said lot, excluding corner lots and vacant lots.

- No fence on any corner lot shall extend beyond the building line or the front of any principal building on any adjacent lot, whichever is less, unless the fence is forty-eight inches (48") or less in height with slatted openings (such as a picket fence or maximum of 50 percent opaque) and has a gated entrance for access by any village authority and/or utility or their contractor to access the easements.

- Fences not over six feet (6') in height may be constructed on any vacant lot only when the record owner of the vacant lot is also the record owner of an adjacent lot improved with a principal building, and then only if the fence to be constructed on the vacant lot does not extend beyond the building setback line of the lot, or beyond the front of any principal building on an adjacent lot, whichever is less.

- Exception for Lakefront Lots: Properties with frontage along a natural lake and consisting of a minimum of eighteen thousand (18,000) square feet as measured above the high water level of the adjacent lake, may have a fence of up to six feet (6') in height along its front and side property lines (Ord. 02-10-42, 10-21-2002).

## GENERAL REQUIREMENTS

No barbed wire, razor wire, or similar material is allowed. Electrically charged fences are prohibited.

The boundaries of the property shall be identified to properly locate the fence and shall remain in place until inspections are completed.  The proper location of the fence is the responsibility of the contractor/homeowner. If the property pins can not be found, a survey is required before installing the fence.

- Attachment to an adjoining fence requires written permission from the owner of the fence.

- All fences shall be wholly within lot lines.

- Fence heights shall not exceed six feet, except that fence height shall not exceed four feet in any front or corner side yard, and fencing shall be 50% opaque (all gaps to be 3 ½”).

- Every fence shall be constructed to resist a horizontal wind pressure of not less than 35 pounds per square foot in addition to other normal forces.

- All support posts must be set in concrete or compacted gravel to a minimum depth of 36 inches.

- End posts are required within your lot line.

- No advertising signs are allowed on fences.

## WHAT INFORMATION DO I NEED TO SUBMIT FOR A PERMIT?

- Completed Permit Application.

- Proposal from Contractor; if applicable.

- Three Scalable Certified Copies of the Plat of Survey, in its entirety, with the proposed location of the fence. Indicate the side and rear set backs, dimensions from nearest lot line to proposed fence, and height (or varying heights where applicable) of the fence.

- Building Plans – materials, type of fence, etc.

- Homeowner’s Association Approval Letter or sign Disclosure (attached) whichever is applicable.

## WHAT HAPPENS NEXT?

We will contact you by phone or email within 5 – 10 business days to inform you when your permit is ready to be picked up. Work cannot be started until the permit is paid for and picked up. The permit must be picked up in person. We accept cash, checks or credit cards.

## HOW MUCH WILL MY PERMIT COST?

Permit fees are determined by the cost of construction (2%), or a minimum of $50.00, which includes plan review fees, and all inspections plus a $12 filing fee.

WHAT INSPECTIONS ARE REQUIRED?

- Post Hole – After all postholes are dug and prior to setting posts.

- Fence Completion.

Inspection are scheduled Monday through Friday by contacting the building department before 2:00 pm the day before the inspection is needed. Provide permit number and address when calling.

IF THE INSPECTOR NEEDS TO RETURN FOR THE SAME INSPECTION, A $150.00 RE-INSPECTION FEE IS REQUIRED TO BE PAID BEFORE A RE-INSPECTION CAN BE SCHEDULED.