HVAC

Below you will find a summary of code requirements for HVAC/Furnace construction within the Village of Antioch, based upon the 2012 International Residential Code.

WHERE CAN THE FURNACE A/C BE LOCATED?

- Refer to the manufacturer’s installation brochure for proper installation on where the furnace may be located including any special installation instructions per the manufacturer’s recommendations.

- If the Building Code differs from the manufacturer’s instructions the manufacturer’s recommendations for installation must be adhered to.

COMMERCIAL  GENERAL REQUIREMENTS

- Damper controls needed on each supply branch

- Minimum of three screws and UL approved foil tape per branch.

- All penetrations must be properly sealed to prevent rodent infestation.

- Copper must be separated from all other dissimilar metals.

- Condensate disposal must run directly to a drain.

- Check for leaks on all gas pipe replaced.

- Appliance must have 30” of open area in front of unit to have access to work on units.

- Vibration isolation must be used between supply and return plenums.

- The wire must match the breaker size.

- The unit nameplate must match breaker size.

- Branch supports for supply gas lines every 10’ maximum

- Roof top units must be set on top of a curb on the roof

- Curb and unit must be properly flashed to prevent leaks on the roof

- A GFI moisture protected outlet must be located within 25’ of unit

WHAT INFORMATION DO I NEED TO SUBMIT FOR A PERMIT?

- Permit Application.

- Proposal from Contractor; if applicable.

- Unit Brochures from manufacturer

## WHAT HAPPENS NEXT?

We will contact you by phone or email within 5 – 10 business days to inform you when your permit is ready to be picked up. Work cannot be started until the permit is paid for and picked up. The permit must be picked up in person. We accept cash, checks or credit card.

## HOW MUCH WILL MY PERMIT COST?

Permit fees are determined by the cost of construction (2%), or a minimum of $50.00 and a $12.00 filing fee, which includes plan review fees and all inspections.

## WHAT INSPECTIONS ARE REQUIRED?

.

- Completion

Inspection are scheduled Monday through Friday by contacting the building department by 2:00pm the day before the inspection is needed. Provide permit number and address when calling.

IF THE INSPECTOR NEEDS TO RETURN FOR THE SAME INSPECTION, A $150.00 RE-INSPECTION FEE IS REQUIRED TO BE PAID BEFORE A RE-INSPECTION CAN BE SCHEDULED.