PATIOS/SIDEWALKS

Below you will find a summary of code requirements for patio construction within the Village of Antioch, based upon the 2012 International Residential Code.

WHERE CAN MY PATIO OR SIDEWALK BE LOCATED?

- Patios must be constructed within the building lines.

- Patios may not be placed in any easements.

- Masonry Patios must maintain a minimum ten-foot (10’) setback.. Rear yard only. Patio MUST be at grade.

- Sidewalks (Maximum four feet (4’) wide and minimum three feet (3’) off the sideyard.

- The patio, sidewalks as well as all other structures on the property, cannot exceed more than 35% of the lot area.

SPECIFIC CONCRETE REQUIREMENTS TO BE NOTED IN YOUR DRAWING

- Plans shall include all measurements of the proposed patio showing lot lines, all adjacent structures and equipment.

- 6 x 6 #10 welded wire mesh.

- 4” of gravel base is required, minimum CA-6, compacted.

- 4” of concrete minimum is required (not 3.5”).  A 6-bag mix per cubic yard, with 3% to 6% air entrainment is required.

- All vegetation must be removed.

- Concrete must not hinder the natural flow of drainage away from foundations.

- Install #4 rebar at least 24” o.c. at house foundation.

- Patio shall be formed with mesh and rebar installed for pre-pour inspection.

- Steps

  - Risers, cannot exceed 7 ¾”

  - Tread Depth, minimum of 10”

SPECIFIC BRICK PAVER REQUIREMENTS TO BE NOTED IN YOUR DRAWING

- Plans shall include all measurements of the proposed patio, driveway and/or walks showing lot lines, all adjacent structures and equipment.

- Specifications indicating the amount of base and the amount of bedding. Pavers require a minimum of 6-12 inches of base and 1-1 ½ inches of bedding.

- Edge restraints must be installed.

- The base, bedding and paver bricks must be compacted.

- If a seat wall is going to be installed, provide additional base below to adjust for added weight.

- If a paver edge is installed adjacent to your existing driveway, it will be considered a non-structural border.

WHAT INFORMATION DO I NEED TO SUBMIT FOR A PERMIT?

- Completed Permit Application.

- Proposal from Contractor; if applicable.

- Three Scalable Certified Copies of the Plat of Survey, in its entirety, showing all existing structures, accessories and flat work, with the proposed location of the patio.

- Three Sets of Building Plans showing all required measurements of the patio with all installation details noted.

- Homeowner’s Association Approval Letter if applicable.

## WHAT HAPPENS NEXT?

We will contact you by phone or e-mail within 5 – 10 business days to inform you when your permit is ready to be picked up. Work cannot be started until the permit is paid for and picked up. The permit must be picked up in person. We accept cash, checks or credit card.

## HOW MUCH WILL MY PERMIT COST?

Permit fees are determined by the cost of construction (2%), or a minimum of $50.00, which includes plan review fees and all inspections, plus a $12.00 filing fee.

## WHAT INSPECTIONS ARE REQUIRED?

- Concrete Patio - Pre-pour with compacted gravel, rebar and mesh in place.

- Paver Bricks – After stone is in place and compacted.

- Completion

Inspection are schedule by calling the building department by 2pm the day before the inspection is needed. Provide permit number and address when calling.

IF THE INSPECTOR NEEDS TO RETURN FOR THE SAME INSPECTION, A $150.00 RE-INSPECTION FEE IS REQUIRED TO BE PAID BEFORE A RE-INSPECTION CAN BE SCHEDULED.