# ROOFS

Below you will find a summary of code requirements for the roofs based upon the 2012 International Residential Code.

## GENERAL REQUIREMENTS

- Re-shingle over 1 layer of existing shingles only. If 2 layers exist, they must be removed before you install the new layer.

- If all the roof sheathing must be replaced it has to be 5/8” and if the rafters/trusses are >16” o.c. use “H” clips. If partial sheathing replacement is necessary, match the existing.

- Use roofing nails long enough to satisfy the manufacturer’s requirements.

- Roof must have adequate ventilation.

- Install an “ice and water shield” type product to extend inwards at least 24” inside the interior of the room.

## WHAT INFORMATION DO I NEED TO SUBMIT FOR A PERMIT?

- Completed Permit Application.

- Proposal from contractor or copy of contract.

- Copy of valid Illinois Roofer’s License (provide with every application).

- Copy of the Homeowner’s Association Approval Letter if applicable.

## WHAT HAPPENS NEXT?

## We will contact you by phone or email within 5 business days to inform you your permit is ready to be picked up. The permit must be picked up in person. Provide property address when picking up permit. Work shall not start until the permit is paid for, picked up and posted. We accept cash, checks or credit cards.

## HOW MUCH WILL MY PERMIT COST?

Permit fees are determined by the cost of construction (2%), or a minimum of $50.00, which includes plan review fees, all inspections and a $12.00 filing fee.

WHAT INSPECTIONS ARE REQUIRED?

- Ice/Water Shield

- Completion

Inspections are scheduled by calling the building department by 2pm the day before the inspection is needed. Provide property address and permit number when calling.