ROOM ADDITIONS

Below you will find a summary of code requirements for room additions based upon the 2012 International Residential Code.

WHERE CAN MY ROOM ADDITION BE LOCATED?

- A room addition built after completion of the main building must maintain single-family residence lot line distances, including rear, sides and building line setbacks.

- Rear yard setbacks are 25’. Room additions cannot extend beyond the side building lines.

- The existing grade and drainage of this property can in no way be changed from the original plan as approved by the Village Engineer.

- The addition, as well as all other structures on the property, cannot exceed more than 35% of the lot area.

## SPECIFIC REQUIREMENTS TO BE NOTED IN YOUR DRAWING

- Light and ventilation schedule. Indicate sizes on drawing.

- Egress window schedule for bedrooms and basement.

- Ceiling heights.

- Header, beam, sawn lumber floor and ceiling joist layouts and sizes.

- Trusses layouts, both roof and floor.

- Door schedule.

- Provide a detailed wall section.

- Provide heating and air conditioning drawing.

- Indicate location of smoke detectors and carbon monoxide detectors.

- Show plumbing.

- Provide mechanical ventilation in bath room.

- Indicate all electric, location of main service panel.

- Show any masonry flashing and lintels.

## WHAT INFORMATION DO I NEED TO SUBMIT FOR PERMIT

- Completed permit application.

- Proposal from contractor.

- Three scalable certified copies of the Plat of Survey, in its entirety, showing the location and size of the proposed addition to scale, including proposed setbacks.

- Three complete sets of building plans.

- Copy of Electrician’s License and Original Insurance Certificate ($1 Million Liability).

- Copy of Plumber’s Illinois License (055, 058).

- Copy of the Homeowner’s Association Approval Letter or sign Disclosure (attached) whichever is applicable.

## WHAT HAPPENS NEXT?

We will contact you by phone or fax within 5 – 10 business days to inform you when your permit is ready to be picked up. Work cannot be started until the permit is paid for and picked up. The permit must be picked up in person. We accept cash or checks. If there are additional items needed before permit issuance we will try to contact you as soon as possible.

## HOW MUCH WILL MY PERMIT COST?

Permit fees are determined by the cost of construction (2%), or a minimum of $50.00, which includes plan review fees and all inspections, plus a $12 filing fee.

WHAT INSPECTIONS ARE REQUIRED?

- Footing

- Backfill

- Rough framing, plumbing, electric and mechanicals

- Insulation

- Completion

Inspections are scheduled Monday through Friday by contacting the building department by 2:00 pm the day before the inspection is needed. Provide permit number and address when calling.

IF THE INSPECTOR NEEDS TO RETURN FOR THE SAME INSPECTION, A $150.00 RE-INSPECTION FEE IS REQUIRED TO BE PAID BEFORE A RE-INSPECTION CAN BE SCHEDULED.